STEP 1. Installing the Control Center (Arduino IDE)

First, we install the program itself.

- Download:

  - Go toarduino.cc/software.

  - Click Windows Win 10 and newer, 64 bits.

  - (If it asks for money, click the grey "JUST DOWNLOAD" button).

- Install: Run the downloaded file and simply click "Next" -> "Next" -> "Finish".

⚠️The most important step for ESP32:

The program itself doesn't know about powerful ESP32 boards. It needs to be taught.

- Open the Arduino IDE.

- In the menu: File → Preferences.

- In the Additional Boards Manager URLs field, paste this link: https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Click OK.

- Go to Tools → Board → Boards Manager.

- Type esp32 in the search and click INSTALL (version from Espressif Systems).

STEP 2. Drivers (So that the computer can see the board)

Connect the board to the USB. If the computer doesn't make a sound ("Ding!"), you need a driver.

- Option A (Normal):Download CP210x.

- Option B (Chinese chip):Download CH340.

How to check? Press Win + X → Device Manager. A device without a yellow triangle should appear in the "Ports (COM)" section.

STEP 3. Installing Libraries (Two Methods)

Method A: "I have a title, I need to find it and download it."

The most common option.

- In the Arduino IDE, press Ctrl + Shift + I (or menu: Sketch → Include Library → Manage Libraries).

- A search box will open on the left. Enter the name (for example, Adafruit BMP280).

- Hover your mouse over the desired line and click INSTALL.

  - ⚠️Important: If asked "Install dependencies?", always click INSTALL ALL.

Method B: "I already have a downloaded .ZIP file"

Use this method if you downloaded the library from GitHub or a flash drive.

- In the Arduino IDE, go to the menu: Sketch → Include Library → Add .ZIP Library...

- A File Explorer window will open. Locate your downloaded file (for example, MQSensorsLib.zip).

- Select it and click Open.

- Done! The message "Library installed" will appear at the bottom.

Objective: To verify that the ESP32-S2 microcontroller is alive, the drivers are installed correctly, and the board can send text messages to the computer via a USB cable.

Code 1: TEST_SERIAL

void setup() {

// 1. Open a communication channel with the computer

// Data transfer rate: 115200 bps

Serial.begin(115200);

// 2. A short pause (2 seconds)

// This is necessary so that you have time to open the port monitor after loading

delay(2000);

// 3. Sending a welcome message once at startup

Serial.println("The ESP32-S2 on-board computer is ready for operation!");

}

void loop() {

// 4. Infinite loop: sending status

Serial.println("Satellite signal: PING");

// 5. Wait 2 seconds before repeating to avoid spamming the screen

delay(2000);

}

Code explanation:

1. Serial.begin(115200); This command starts the UART (Serial) interface. The number 115200 is the baud rate. It's important that the same number is selected in the Serial Monitor settings on your computer, otherwise you'll see gibberish instead of text.

2. Serial.println("..."); The command to print text. The ln (line) at the end means that after the text, the cursor will move to a new line (like pressing Enter). If you simply use Serial.print, the text will merge into one long line.

3. delay(2000); Pause command. The microcontroller pauses program execution for the specified number of milliseconds (2000 ms = 2 seconds). Without this command, the text would fly by at incredible speed, making it impossible to read.

Code 2: LED_BLINK

Objective: To learn how to control the microcontroller outputs (GPIO) by applying voltage to the built-in LED to create a visual signal.

Code 2: LED_BLINK (Beacon)

// According to the S2 Mini pinout, the onboard LED is located on GPIO 15

#define LED_PIN 15

void setup() {

// Configure pin 15 as "OUTPUT".

// This means that we will be supplying voltage from it, not reading it.

pinMode(LED_PIN, OUTPUT);

}

void loop() {

// 1. Turn on the LED

// HIGH = "High level" (3.3 Volts supplied)

digitalWrite(LED_PIN, HIGH);

// 2. Wait 500 milliseconds (half a second)

delay(500);

// 3. Turn off the LED

// LOW = "Low level" (voltage 0 volts)

digitalWrite(LED_PIN, LOW);

// 4. Wait another 500 milliseconds before repeating the loop

delay(500);

}

Code explanation:

1. #define LED_PIN 15: We give pin 15 a descriptive name, LED_PIN. On the S2 Mini board, the onboard blue LED is physically connected to pin 15. If we were using a different board, the number might be different (for example, GPIO 2 on a classic ESP32).

2. pinMode(LED_PIN, OUTPUT);: This command puts the processor pin into "power source" mode. Without this line, the LED will glow dimly or not light at all, as the pin will not be able to supply sufficient current.

3. digitalWrite(LED_PIN, HIGH/LOW);: This is the "switch".

◦ HIGH: The controller opens the damper and supplies 3.3V to the LED - the light is on.

◦ LOW: The controller removes the voltage (grounds the pin) - the light goes out.

4. delay(500);: Causes the processor to "freeze" for the specified number of milliseconds. If this command is omitted, the LED will turn on and off millions of times per second, and the eye will see this as a dim glow rather than blinking.

Code 3: LED_MORSE (“Morse code”)

Goal: To teach the satellite to autonomously communicate its status (e.g., "Everything is OK" or "Help Needed") using only one LED. This is critical for field diagnostics when there is no computer connection.

In this code, we use pin 15 (the built-in LED on the S2 Mini board). After the satellite is fully assembled on the Main Board, this LED (LED1) will be located on pin 37.

Code 3: LED_MORSE (Beacon)

// LED pin (15 - built-in on S2 Mini, 37 - LED1 on the main board)

const int LED_PIN = 15;

// Timings (in milliseconds) according to the course standard

const int DOT_TIME = 200; // Dot (short flash)

const int DASH_TIME = 600; // Dash (long flash, 3 dots)

const int GAP_TIME = 200; // Pause between signals

const int LETTER_GAP = 600; // Pause between letters

const int WORD_GAP = 1500; // Pause between words

void setup() {

pinMode(LED_PIN, OUTPUT);

}

// Function for flash "POINT" (.)

void dot() {

digitalWrite(LED_PIN, HIGH);

delay(DOT_TIME);

digitalWrite(LED_PIN, LOW);

delay(GAP_TIME);

}

// Function for the "DASH" (-) flash

void dash() {

digitalWrite(LED_PIN, HIGH);

delay(DASH_TIME);

digitalWrite(LED_PIN, LOW);

delay(GAP_TIME);

}

// SOS signal transmission function (... --- ...)

void blinkSOS() {

// S (...)

dot(); dot(); dot();

delay(LETTER_GAP);

// O (---)

dash(); dash(); dash();

delay(LETTER_GAP);

// S (...)

dot(); dot(); dot();

delay(WORD_GAP);

}

// OK signal transmission function (--- -.-)

void blinkOK() {

// O (---)

dash(); dash(); dash();

delay(LETTER_GAP);

// K (-.-)

dash(); dot(); dash();

delay(WORD_GAP);

}

void loop() {

// The satellite transmits statuses in turn

blinkOK(); // We report "OK"

delay(2000); // Pause

blinkSOS(); // Send "SOS"

delay(3000); // Long pause before repeating

}

Code explanation:

1. The dot() and dash() functions: Instead of repeatedly writing on and off commands (digitalWrite), we created separate functions for the "dot" and "dash." This is called task decomposition—breaking a complex process into simple, repeatable actions.

2. Timings: We moved the signal durations into variables (const int) at the beginning of the code.

◦ Point = 200 ms.

◦ Dash = 600 ms (3 times longer than dot).

◦ This makes it easy to change the baud rate throughout the entire program by simply changing the numbers in one place.

3. Loop() logic: In an infinite loop, the satellite transmits an "OK" signal, waits for 2 seconds, then transmits an "SOS" signal and waits for 3 seconds. In real flight code, the logic would change: the satellite would transmit blinkOK() only if all systems are operational, or blinkSOS()/blinkFAIL() if an error was detected.

Code 4: I2C_SCANNER ("Hardware Scanner")

Goal: Check the physical connection of all sensors to the shared I2C data bus. This code loops through all possible "addresses" (from 1 to 127) and checks if anyone responds to them.

Code 4: I2C Scanner

#include <Wire.h> // Library for working with the I2C protocol

void setup() {

// 1. Initializing the I2C bus

// IMPORTANT: For ESP32-S2 Mini, specify pins SDA = 1, SCL = 2

Wire.begin(1, 2);

Serial.begin(115200); // Start communication with the PC

while (!Serial); // Wait for the serial monitor to open

Serial.println("\n--- I2C SCANNER STARTED ---");

}

void loop() {

byte error, address;

int nDevices = 0;

Serial.println("Scanning...");

// 2. Cycle through all addresses

for (address = 1; address < 127; address++) {

// Start transmitting to the address

Wire.beginTransmission(address);

// We complete the transmission and see if a response (ACK) has arrived.

error = Wire.endTransmission();

// 3. Response processing

if (error == 0) { // If the device responded "I'm here"

Serial.print("Device found at address: 0x");

if (address < 16) Serial.print("0");

Serial.println(address, HEX); // Print the address in hexadecimal format

nDevices++;

}

}

if (nDevices == 0)

Serial.println("No devices found. Check wires (SDA/SCL)!");

else

Serial.println("Scanning complete.\n");

delay(5000); // Repeat the search every 5 seconds

}

Code explanation:

1. Wire.begin(1, 2);: The most critical line for this board. The standard Wire.begin() command searches for sensors on other pins (usually 21 and 22). For the ESP32-S2 Mini in your kit, the pinout is on pins 1 (SDA) and 2 (SCL). Without explicitly specifying these numbers, the scanner won't find anything.

2. Wire.beginTransmission(address) and Wire.endTransmission(): This is analogous to "knocking on the door." The controller sends a signal to a specific address. If a sensor is present at that address, it must send an acknowledgement (ACK).

◦ If error == 0, it means that the sensor heard its address and responded (pulled the data line to ground).

3. Result: In the port monitor, you should see the addresses of the connected devices. The following are expected for your set:

◦ 0x68 — Gyroscope/Accelerometer (MPU6050).

◦ 0x76 - Barometer (BMP280).

◦ 0x40 — Current sensor (INA219).

◦ 0x0D or 0x1E — Magnetometer (QMC5883L/HMC5883L).

◦ 0x29 — Laser rangefinder (VL53L0X)

Code 5: MPU_BMP_TEST ("Orientation and Atmosphere")

Objective: To teach the satellite to read the physical parameters of the environment: temperature, atmospheric pressure (altitude) and position in space (acceleration/tilt).

This code combines the work of two sensors connected to the same I2C bus:

1. BMP280 (Barometer/Thermometer) - address 0x76.

2. MPU6050 (Accelerometer/Gyroscope) - address 0x68.

Code 5: MPU_BMP_TEST

#include <Wire.h>

#include <Adafruit_BMP280.h> // Pressure and temperature library

#include <Adafruit_MPU6050.h> // Library for the accelerometer and gyroscope

#include <Adafruit_Sensor.h>

Adafruit_BMP280 bmp; // Create a "bmp" object to work with the sensor

Adafruit_MPU6050 mpu; // Create an "mpu" object for orientation

void setup() {

Serial.begin(115200);

// 1. Initializing the I2C bus on specific S2 Mini pins

// SDA = 1, SCL = 2

Wire.begin(1, 2);

Serial.println("Searching for sensors...");

// 2. Starting the BMP280 barometer (address 0x76)

if (!bmp.begin(0x76)) {

Serial.println("ERROR: BMP280 not found! Check soldering/address.");

while (1); // Infinite loop (stop) if sensor is broken

}

// 3. Starting the MPU6050 gyroscope (default address 0x68)

if (!mpu.begin()) {

Serial.println("ERROR: MPU6050 not found!");

while (1);

}

Serial.println("Sensors are ready to work!");

}

void loop() {

// 4. Receiving data from MPU6050

// For this sensor, you need to create "events" (variables) where it will write numbers

sensors_event_t a, g, temp;

mpu.getEvent(&a, &g, &temp);

// 5. Reading data from BMP280

float temperature = bmp.readTemperature();

float pressure = bmp.readPressure();

// 6. Output to the port monitor

Serial.print("Temp: ");

Serial.print(temperature);

Serial.print(" C | ");

Serial.print("Pressure: ");

Serial.print(pressure / 100.0); // Divide by 100 to convert Pascals to Hectopascals (hPa)

Serial.print(" hPa | ");

Serial.print("Accel Z: ");

Serial.print(a.acceleration.z); // Acceleration along the vertical axis

Serial.println(" m/s^2");

delay(500); // Update data twice per second

}

Code explanation:

1. Wire.begin(1, 2);: Just like in the scanner, we force the controller to connect the physical data wires (SDA and SCL) to pins 1 and 2. Without this command, the Adafruit libraries will look for sensors on the standard pins (21/22) and find nothing.

2. mpu.getEvent(&a, &g, &temp);: The MPU6050 sensor outputs a "batch" of data: acceleration (a), rotation (g), and its temperature (temp). We pass variable references to the function, and it populates them with the current values. a.acceleration.z represents the force of gravity. If the board is lying flat on a table, this value will be around 9.8 m/s² (the force of gravity is 1G).

3. bmp.readPressure() / 100.0: The sensor outputs pressure in Pascals (e.g., 101325 Pa). In meteorology, hectopascals (millibars) are commonly used. Dividing by 100 converts this number to the familiar ~1013 hPa.

4. while (1);: This is an "emergency stop." If the controller doesn't find the sensor during setup, the program gets stuck at this line forever. This prevents junk data (zeros) from being sent to the main loop.

Code 6: INA219_POWER ("Energy Budget")

Objective: To teach the satellite to independently measure the charge level of its batteries (voltage) and the rate of energy consumption (current and power).

This code uses the INA219 sensor mounted on the Power Board. It functions as a "power meter" for the satellite.

Code 6: INA219_POWER (Power Budget)

#include <Wire.h>

#include <Adafruit_INA219.h>

// Create a sensor object (default address 0x40)

Adafruit_INA219 ina219;

void setup() {

Serial.begin(115200);

// !!! CRITICAL FOR CUBESAT KIT (S2 ​​Mini) !!!

// Reassign I2C pins to 1 (SDA) and 2 (SCL)

Wire.begin(1, 2);

Serial.println("Initializing INA219 power sensor...");

// Checking the sensor connection

if (!ina219.begin()) {

Serial.println("ERROR: Power sensor not found!");

Serial.println("Check the PH2.54 connection between the Power and Main boards.");

while (1); // Hang (stop the program) if there is no sensor

}

Serial.println("Power: OK. Starting measurements...");

}

void loop() {

// 1. Reading physical quantities

float bus_V = ina219.getBusVoltage_V(); // Battery voltage (V)

float current_mA = ina219.getCurrent_mA(); // Current consumption (mA)

float power_mW = ina219.getPower_mW(); // Power (mW) = Voltage * Current

// 2. Calculating the percentage of charge (linear approximation for Li-ion 2S or 1S)

// For one 18650 cell: 4.2V = 100%, 3.0V = 0%

// Note: If you have a serial connection (2S), the values ​​will be ~8.4V

float charge_pct = (bus_V - 3.0) / (4.2 - 3.0) * 100;

// Limit the boundaries to 0..100%

if (charge_pct > 100) charge_pct = 100;

if (charge_pct < 0) charge_pct = 0;

// 3. Outputting data to the Serial Monitor

Serial.print("Bat: ");

Serial.print(bus_V);

Serial.print(" V (");

Serial.print((int)charge_pct);

Serial.print("%) | ");

Serial.print("Current: ");

Serial.print(current_mA);

Serial.print(" mA | ");

Serial.print("Power: ");

Serial.print(power_mW);

Serial.println(" mW");

delay(1000); // Update data once per second

}

Code explanation:

1. ina219.getBusVoltage_V(): This function measures the "Bus Voltage" - the voltage in the power supply circuit.

◦ If powered by USB: will show about 4.5–5.0 V.

◦ If from a battery (18650): it will show 3.7–4.2 V (for one cell) or higher if the circuit is 2S.

2. ina219.getCurrent_mA(): Measures the current flowing through a special resistor (shunt) on the power board.

◦ The value can be negative or positive depending on the direction of the current, but the library usually returns a positive consumption value.

◦ The current is measured through the voltage drop across a 0.1 Ohm shunt (R100).

3. ina219.getPower_mW(): Calculates instantaneous power using the physics formula. This shows how quickly the satellite "eats" the battery.

4. Calculating the percentage of charge: The code uses a simplified linear formula. In reality, Li-ion batteries discharge nonlinearly (holding 3.7V for a long time, then dropping sharply), but this is sufficient for a basic assessment.

🛠What to check (Checklist):

• “0.00 mA”: If the current is zero, but the satellite is working, the INA219 sensor may not be calibrated or the power supply may be bypassing the sensor (for example, only via USB, and the sensor is on the battery line).

• Changing the current: Try turning on the LEDs (code from previous lessons) or Wi-Fi - the Current value should increase.

Code 7: VL53L0X_TEST ("Laser rangefinder")

Goal: To teach the satellite to measure distances to objects using a laser. This sensor is located on the Vision Board (Payload).

Library: You will need to install Adafruit_VL53L0X via the Arduino IDE library manager.

Code 7: VL53L0X_TEST (Laser rangefinder)

#include <Wire.h>

#include <Adafruit_VL53L0X.h>

// Create a rangefinder object

Adafruit_VL53L0X lox = Adafruit_VL53L0X();

void setup() {

Serial.begin(115200);

// Initialize the I2C bus for the ESP32-S2 Mini

// Important: On the CubeSat kit boards, the sensors are located on pins 1 and 2

Wire.begin(1, 2); // SDA=1, SCL=2

Serial.println("Starting VL53L0X test...");

// Attempt to start the sensor (default address 0x29)

if (!lox.begin()) {

Serial.println("ERROR: VL53L0X not found!");

Serial.println("Check Payload board power and I2C connection.");

while (1); // Stop the program on error

}

Serial.println("VL53L0X is ready for operation");

}

void loop() {

// Create a variable to store measurement data

VL53L0X_RangingMeasurementData_t measure;

// We make a measurement

lox.rangingTest(&measure, false); // false = no extra debugging information

// Check status (4 = error or out of range)

if (measure.RangeStatus != 4) {

Serial.print("Distance: ");

Serial.print(measure.RangeMilliMeter);

Serial.println(" mm");

} else {

Serial.println("Out of range (Too far or no reflection)");

}

delay(300); // A short pause between measurements

}

Code explanation:

1. Operating principle (ToF): The sensor uses Time of Flight technology. It emits an invisible laser pulse (940 nm), which is reflected off an obstacle and returned back. The sensor measures this travel time and calculates the distance.

2. lox.rangingTest(&measure, false);: This command starts the measurement process. The results are written to the measure structure. The sensor operates in a range from 30 mm to 2000 mm (2 meters).

3. measure.RangeStatus != 4: The sensor may return an error if the object is too far away or the surface is black (poorly reflective).

◦ If the status is not 4, then the measurement was successful, and we output measure.RangeMilliMeter (distance in millimeters).

◦ If the status is 4, the message "Out of range" is displayed.

4. Connection: The sensor is connected to the common I2C bus. In the code, we must specify Wire.begin(1, 2), as the ESP32-S2 Mini board in this kit uses these pins for SDA and SCL.

Code 8: MQ_GAS_TEST ("Gas Analysis")

Objective: To teach the satellite to read readings from three analog gas sensors (MQ-2, MQ-8, MQ-9) and trigger an alarm when a dangerous concentration is detected.

Important: The gas sensors operate at 5V, but through a voltage divider, they supply a safe signal (up to 3.3V) to the ESP32. To function properly, they require heating (the sensors will be warm to the touch) and a burn-in period (1–2 minutes after power-on).

Code 8: MQ_GAS_TEST (Gas Analysis and Alarm)

// 1. PIN SETTINGS (According to connection map v5.6)

// Attention: Check the pinout of your board version!

// In v5.5, pins 3, 4, 15 can be used.

const int PIN_MQ2 = 4; // Smoke / Propane / Methane

const int PIN_MQ8 = 5; // Hydrogen

const int PIN_MQ9 = 6; // Carbon monoxide (CO)

// Alarm threshold (0..4095)

// If the value is higher, we assume there is a leak

const int THRESHOLD = 2500;

void setup() {

Serial.begin(115200);

// 2. SETTING UP THE ADC (Analog-to-Digital Converter)

// ESP32 has 12-bit resolution (numbers from 0 to 4095)

// Unlike Arduino (10 bits, 0..1023)

analogReadResolution(12);

// Setting up pins for input

pinMode(PIN_MQ2, INPUT);

pinMode(PIN_MQ8, INPUT);

pinMode(PIN_MQ9, INPUT);

Serial.println("--- GAS ANALYSIS SYSTEM STARTED ---");

Serial.println("Wait 2 minutes for sensors to warm up...");

// Table header for ease of reading

Serial.println("MQ-2\tMQ-8\tMQ-9\tSTATUS");

}

void loop() {

// 3. DATA COLLECTION

// Read the voltage on the pins (returns the number 0..4095)

int val2 = analogRead(PIN_MQ2);

int val8 = analogRead(PIN_MQ8);

int val9 = analogRead(PIN_MQ9);

// 4. SERIAL OUTPUT

// \t is a tab stop for even columns

Serial.print(val2); Serial.print("\t");

Serial.print(val8); Serial.print("\t");

Serial.print(val9); Serial.print("\t");

// 5. LOGIC OF ANXIETY

// If at least one sensor has exceeded the threshold

if (val2 > THRESHOLD || val8 > THRESHOLD || val9 > THRESHOLD) {

Serial.println("!!! ALARM!!!");

} else {

Serial.println("Normal");

}

delay(1000); // Update data once per second

}

Code explanation:

1. analogReadResolution(12);: A critical command for the ESP32. A standard Arduino divides the voltage into 1024 steps (10 bits). The ESP32 can divide more precisely—into 4096 steps (12 bits).

◦ 0 Volt = number 0.

◦ 3.3 Volts = number 4095.

2. Sensor Physics (Chemiresistor): The sensor operates as a variable resistor. It contains a heater inside.

◦ Clean air: High resistance → Low voltage → Small number in the code (e.g. 500–1200).

◦ Gas: Resistance drops → Voltage rises → Number approaches 4095.

3. Alarm logic (if ... || ...): We use the OR logical operator (||). The alarm will be triggered if the threshold is exceeded on the MQ-2 OR on the MQ-8 OR on the MQ-9. This ensures safety in the event of a gas leak of any type.

Code 9: GPS_RAW ("Searching for satellites")

Objective: To obtain real geographic coordinates (latitude, longitude) and altitude above sea level using the NEO-6M module.

Important: GPS is a passive system; the module only "listens" to signals. For the first startup (a "cold start"), the antenna must be placed near a window or outside; satellite acquisition may take 3 to 5 minutes.

Code 9: GPS_RAW (Searching for satellites)

Library: TinyGPS++

#include <TinyGPS++.h>

// Setting up pins for the ESP32-S2 Mini

// RX GPS connects to TX ESP (assign 17)

// TX GPS connects to RX ESP (assign 18)

#define GPS_RX 18

#define GPS_TX 17

#define GPS_BAUD 9600 // Standard baud rate of the NEO-6M module

TinyGPSPlus gps; // Create an object for working with data

void setup() {

Serial.begin(115200);

// Enable hardware Serial1 to communicate with GPS

// Parameters: Speed, Mode (8 bits, no parity, 1 stop bit), RX pin, TX pin

Serial1.begin(GPS_BAUD, SERIAL_8N1, GPS_RX, GPS_TX);

Serial.println("Searching for satellites... (Take the antenna outside)");

Serial.println("Wait for the PPS LED on the module to blink.");

}

void loop() {

// Read data from the GPS port while it's coming in

while (Serial1.available() > 0) {

// Feed bytes to the library to decode the NMEA protocol

if (gps.encode(Serial1.read())) {

// Check if there are valid (reliable) coordinates

if (gps.location.isValid()) {

Serial.print("LAT: ");

Serial.print(gps.location.lat(), 6); // Latitude (6 decimal places)

Serial.print(" | LON: ");

Serial.print(gps.location.lng(), 6); // Longitude

Serial.print(" | ALT: ");

Serial.print(gps.altitude.meters()); // Altitude in meters

Serial.print("m | SATS: ");

Serial.println(gps.satellites.value()); // Number of satellites

} else {

// If satellites have not yet been found

Serial.print("No satellites found. I see: ");

Serial.println(gps.satellites.value());

}

}

}

}

Code explanation:

1. Serial1.begin(9600, ...): The GPS module communicates via the UART protocol. We use the second ESP32 hardware port (Serial1) to avoid interfering with debugging via USB (Serial). 9600 baud is the factory default for the u-blox NEO-6M.

2. gps.encode(Serial1.read()): The GPS module constantly sends raw NMEA text strings (e.g., $GPGGA...). This command collects these characters and converts them into understandable numbers.

3. gps.location.isValid(): Validation check ("fix"). If the module sees less than 3-4 satellites, it cannot calculate coordinates, and this function will return false. Once a fix is ​​received, the PPS LED on the module will start blinking once per second.

4. Coordinates (Float): Latitude and longitude are displayed with an accuracy of up to 6 decimal places (lat(), 6), which gives a positioning accuracy of about 5 meters

Code 10: LORA_BEACON

Objective: To test the functionality of the Ra-02 (SX1278) radio module and send the first data packet. The satellite will cyclically send a "PING" message indicating the uptime.

ATTENTION: Before uploading the code, be sure to connect the antenna (spring) to the ANT connector on the communication board. Enabling LoRa without an antenna can instantly destroy the transmitter, as the energy has nowhere to go.

Code 10: LORA_BEACON (Radio Beacon)

Board: ESP32-S2 Mini

Library: LoRa (sandeep mystery)

#include <SPI.h>

#include <LoRa.h>

// Configure SPI pins for CubeSat kit (ESP32-S2 Mini)

// These pins are different from the standard ones for ESP32, so we set them manually

#define LORA_SCK 12

#define LORA_MISO 13

#define LORA_MOSI 11

#define LORA_SS 10

#define LORA_RST 9

#define LORA_DIO0 8

// Frequency: 433E6 (433 MHz) for Europe/CIS or 868E6/915E6 for other regions

#define BAND 433E6

void setup() {

Serial.begin(115200);

Serial.println("Starting LoRa transmitter...");

// 1. Initialize the SPI bus with specific pins

SPI.begin(LORA_SCK, LORA_MISO, LORA_MOSI, LORA_SS);

// 2. Setting up service pins of the Ra-02 module

LoRa.setPins(LORA_SS, LORA_RST, LORA_DIO0);

// 3. Launching the radio module at a frequency of 433 MHz

if (!LoRa.begin(BAND)) {

Serial.println("ERROR: LoRa not found!");

Serial.println("Check the connection between the Comms board and Main.");

while (1); // Stop the program in case of an accident

}

Serial.println("LoRa is running! Starting broadcast...");

}

void loop() {

Serial.print("Sending packet... ");

// 4. Forming and sending a packet

LoRa.beginPacket(); // Open the "envelope"

LoRa.print("CUBESAT_PING_"); // Write text

LoRa.print(millis() / 1000); // Add time in seconds

LoRa.endPacket(); // Seal and send on air

Serial.println("OK");

delay(5000); // 5 second pause between signals

}

Code explanation:

1. SPI.begin(...) and LoRa.setPins(...): The LoRa module communicates with the processor via the SPI protocol. This kit uses non-standard ESP32 pins (10, 11, 12, 13), so we explicitly tell the controller where to find the radio module.

2. LoRa.begin(BAND): This command initializes the SX1278 chip. If the chip does not respond (bad connection, no power, or burned out), the function will return false, and the program will report an error.

3. LoRa.beginPacket() ... LoRa.endPacket(): The sending process is similar to writing a letter.

◦ beginPacket(): Start collecting data into the buffer.

◦ print(): Add information (text, numbers) to the buffer.

◦ endPacket(): The "Fire!" command. The chip modulates the signal and sends it through the antenna into space.

4. Result: The serial monitor will display "Sending packet... OK" every 5 seconds. This means the microcontroller has successfully transmitted data to the radio chip, which has emitted it. To view this data, you will need a second identical module (receiver) connected to a different computer.

Code 11: GPS_LORA_TRACKER ("Satellite tracker")

Goal: To integrate navigation and communication systems. The satellite must independently obtain its coordinates from space (GPS) and immediately transmit them to Earth via radio (LoRa). This turns the CubeSat into an autonomous beacon tracker.

Important: Make sure the GPS and LoRa antennas are connected. It's best to place the GPS antenna near a window.

Code 11: GPS_LORA_TRACKER

Libraries: TinyGPSPlus, LoRa

#include <TinyGPS++.h>

#include <SPI.h>

#include <LoRa.h>

// --- GPS SETTINGS (UART) ---

// Connect the RX pin of the GPS module to pin 17 (TX) of the ESP32

// Connect the TX pin of the GPS module to pin 18 (RX) of the ESP32

#define GPS_RX 18

#define GPS_TX 17

#define GPS_BAUD 9600 // NEO-6M standard speed

// --- LoRa SETTINGS (SPI) ---

// Pins for ESP32-S2 Mini (check if they match your build)

#define LORA_SCK 12

#define LORA_MISO 13

#define LORA_MOSI 11

#define LORA_SS 10

#define LORA_RST 9

#define LORA_DIO0 8

#define LORA_BAND 433E6 // Frequency 433 MHz

// Create objects

TinyGPSPlus gps;

HardwareSerial GPS_Serial(1); // Use hardware Serial1

void setup() {

Serial.begin(115200); // For debugging via USB

// 1. Launch GPS

// Serial1: RX=18, TX=17

GPS_Serial.begin(GPS_BAUD, SERIAL_8N1, GPS_RX, GPS_TX);

Serial.println("GPS module started. Waiting for satellites...");

// 2. Launch LoRa

SPI.begin(LORA_SCK, LORA_MISO, LORA_MOSI, LORA_SS);

LoRa.setPins(LORA_SS, LORA_RST, LORA_DIO0);

if (!LoRa.begin(LORA_BAND)) {

Serial.println("ERROR: LoRa not found!");

while (1); // Stop on crash

}

Serial.println("LoRa radio channel is active.");

}

void loop() {

// 3. Reading data from GPS

// This loop must run continuously to avoid missing bytes from the satellite

while (GPS_Serial.available() > 0) {

// Scaramelize the data to the library. If the package is fully assembled -> true

if (gps.encode(GPS_Serial.read())) {

// 4. Checking coordinate updates

if (gps.location.isUpdated()) {

Serial.print("Coordinates updated! Latitude: ");

Serial.println(gps.location.lat(), 6);

// 5. Sending a packet into the air

sendLoRaPacket();

}

}

}

}

// Function for forming and sending a packet

void sendLoRaPacket() {

LoRa.beginPacket();

// Packet header

LoRa.print("SAT_POS:");

// Latitude data

LoRa.print(gps.location.lat(), 6);

LoRa.print(",");

// Longitude data

LoRa.print(gps.location.lng(), 6);

// Add height if any

LoRa.print(",ALT:");

LoRa.print(gps.altitude.meters());

LoRa.endPacket();

Serial.println("-> Packet sent via LoRa");

}

Code explanation:

1. Two data streams: The ESP32 acts as a "bridge." On one side, raw data from the GPS module comes in via wires (UART, pins 17/18). On the other side, the processor controls the LoRa radio module via SPI (pins 8-13).

2. gps.encode(GPS_Serial.read()): The GPS module sends data continuously (as an NMEA stream). This function assembles the disjointed characters into understandable numbers (latitude, longitude). It's important to call it in a while loop to prevent the buffer from overflowing.

3. if (gps.location.isUpdated()): We don't send a radio signal every millisecond, but only when the GPS module has actually updated its coordinates (usually once per second, 1 Hz). This saves energy and prevents cluttering the airwaves with duplicates.

4. Packet format: We form a string of the following type: SAT_POS:43.123456,76.987654,ALT:500.2.

◦ The receiver on Earth will see this text and be able to immediately plot the satellite's route on a map. An accuracy of 6 decimal places provides positioning accuracy of up to ~5 meters.

Code 12: TELEMETRY_SERIAL ("Data Collection (CSV)")

The goal: to integrate all sensors into a single system. The satellite should synchronously poll the barometer, gyroscope, and gas analyzer, and then send the data to a computer in CSV (Comma-Separated Values) format, so it can be easily opened in Excel and plotted.

Code 12: TELEMETRY_SERIAL

#include <Wire.h>

#include <Adafruit_BMP280.h>

#include <Adafruit_MPU6050.h>

// --- EQUIPMENT SETUP ---

Adafruit_BMP280 bmp; // Pressure sensor object

Adafruit_MPU6050 mpu; // Motion sensor object

#define PIN_GAS 4 // Pin of the analog gas sensor

void setup() {

Serial.begin(115200);

// Initialize I2C for ESP32-S2 (SDA=1, SCL=2)

Wire.begin(1, 2);

Serial.println("--- LOADING ON-BOARD COMPUTER ---");

// 1. Checking BMP280

if (!bmp.begin(0x76)) {

Serial.println("ERROR: BMP280 sensor not found! Check the wires.");

while (1); // Stop the car (emergency stop)

}

// 2. Checking MPU6050

if (!mpu.begin()) {

Serial.println("ERROR: MPU6050 sensor not found! Check the wires.");

while (1); // Stop the car

}

// 3. Setting up the ADC for gas

analogReadResolution(12); // Enable 0..4095 mode

pinMode(PIN_GAS, INPUT);

Serial.println("ALL SYSTEMS ARE NORMAL. START RECORDING.");

// PRINTING THE TABLE HEADER

// These are the names of our columns for Excel

Serial.println("Time(ms);Temp(C);Press(Pa);Accel_Z;Gas(Raw)");

}

void loop() {

// --- STEP 1: DATA COLLECTION (Acquisition) ---

unsigned long timeStamp = millis(); // Time from start

float temp = bmp.readTemperature(); // Temperature

float press = bmp.readPressure(); // Pressure

int gas = analogRead(PIN_GAS); // Gas

// MPU requires a special event structure

sensors_event_t a, g, t;

mpu.getEvent(&a, &g, &t);

float accelZ = a.acceleration.z; // Vertical acceleration

// --- STEP 2: SENDING TO PORT (CSV Format) ---

// We assemble a string separated by semicolons

Serial.print(timeStamp);

Serial.print(";");

Serial.print(temp);

Serial.print(";");

Serial.print(press);

Serial.print(";");

Serial.print(accelZ);

Serial.print(";");

Serial.println(gas); // At the end of println to go to a new line

// --- STEP 3: RHYTHM OF WORK ---

delay(1000); // Write data exactly once per second

}

Code explanation:

1. CSV (Comma-Separated Values) format: We display data not as words ("The temperature is..."), but strictly separated by a semicolon (;). This is a standard data exchange protocol. A computer or program (such as Excel) understands: "Everything before the first semicolon is the time, the second is the temperature," and so on.

2. Table Header: In setup() we send the string Time(ms);Temp(C);... once. This is necessary so that when copying data into the table the columns immediately have names.

3. Architecture of the loop() cycle: The program works as a pipeline of three stages:

◦ Acquisition: The processor quickly polls all sensors and stores the values ​​in temporary variables (temp, press, gas).

◦ Packetizing: Variables are glued together into a single string with separators.

◦ Send: Serial.println() sends the completed string to the computer.

4. Time Stamp Synchronization: Each line begins with millis()—the time in milliseconds since power-on. This is critical so that you can later plot graphs of changes over time, rather than just seeing a series of numbers.

Code 13: PROJECT_FLIGHT_LORA ("Flight Controller")

Goal: This is the final firmware. It combines everything we've learned: the satellite automatically checks its systems during launch (POST), cyclically collects data from all sensors (GPS, MPU, BMP, INA219, GAS, Laser), packages it into a single packet, and sends it to Earth via the LoRa radio channel. An "Emergency Status" system has also been added for critical situations.

Code 13: PROJECT_FLIGHT_LORA (Final Flight Controller)

#include <Wire.h>

#include <SPI.h>

#include <LoRa.h>

#include <TinyGPS++.h>

#include <Adafruit_BMP280.h>

#include <Adafruit_MPU6050.h>

#include <Adafruit_INA219.h>

#include <Adafruit_VL53L0X.h>

// --- 1. PIN SETTINGS (HARDWARE CONFIG) ---

// I2C for ESP32-S2 Mini (Required 1 and 2!)

#define SDA_PIN 1

#define SCL_PIN 2

// LoRa (SPI) - Custom Pins for S2 Mini

#define LORA_SCK 12

#define LORA_MISO 13

#define LORA_MOSI 11

#define LORA_SS 10

#define LORA_RST 9

#define LORA_DIO0 8

#define LORA_BAND 433E6 // Frequency 433 MHz

// GPS (UART1)

#define GPS_RX 18

#define GPS_TX 17

// Analog sensors and indication

#define PIN_GAS 4 // Gas sensor (MQ-2)

#define LED_STATUS 15 // On-board LED

// --- 2. GLOBAL OBJECTS ---

Adafruit_BMP280 bmp;

Adafruit_MPU6050 mpu;

Adafruit_INA219 ina219;

Adafruit_VL53L0X lox = Adafruit_VL53L0X();

TinyGPSPlus gps;

HardwareSerial gpsSerial(1); // Use hardware Serial1

// --- 3. STATUS FLAGS (SYSTEM HEALTH) ---

// If the sensor fails the test at startup, the flag will become false,

// and we will stop wasting time on his survey.

bool statusBMP = false;

bool statusMPU = false;

bool statusINA = false;

bool statusLOX = false;

bool statusLORA = false;

unsigned long packetCount = 0; // Telemetry packet number

void setup() {

Serial.begin(115200);

// LED settings

pinMode(LED_STATUS, OUTPUT);

digitalWrite(LED_STATUS, HIGH); // Lights while loading

// Run the I2C bus on the correct pins of the S2 Mini

Wire.begin(SDA_PIN, SCL_PIN);

Serial.println("\n=== STARTING THE CUBESAT ON-BOARD COMPUTER ===");

Serial.println(">>> EXECUTING POST (Power-On Self-Test)...");

// --- STEP A: CHECKING SENSORS (POST) ---

// 1. Barometer BMP280 (0x76)

if (bmp.begin(0x76)) {

statusBMP = true;

Serial.println("[OK] BMP280 Atmosphere");

} else {

Serial.println("[FAIL] BMP280 (Connection Error)");

}

// 2. Gyroscope MPU6050 (0x68)

if (mpu.begin()) {

statusMPU = true;

Serial.println("[OK] MPU6050 Orientation");

} else {

Serial.println("[FAIL] MPU6050");

}

// 3. INA219 Power (0x40)

if (ina219.begin()) {

statusINA = true;

Serial.println("[OK] INA219 Power System");

} else {

Serial.println("[FAIL] INA219");

}

// 4. Laser rangefinder (0x29)

if (lox.begin()) {

statusLOX = true;

Serial.println("[OK] VL53L0X Rangefinder");

} else {

Serial.println("[FAIL] VL53L0X");

}

// 5. GPS (UART)

gpsSerial.begin(9600, SERIAL_8N1, GPS_RX, GPS_TX);

Serial.println("[OK] GPS Serial Port");

// 6. Gas analysis (ADC)

analogReadResolution(12);

pinMode(PIN_GAS, INPUT);

Serial.println("[OK] Gas Sensor");

// --- STEP B: STARTING THE RADIO (LORA) ---

// Critical system. If the radio is down, the mission is failed.

SPI.begin(LORA_SCK, LORA_MISO, LORA_MOSI, LORA_SS);

LoRa.setPins(LORA_SS, LORA_RST, LORA_DIO0);

if (LoRa.begin(LORA_BAND)) {

statusLORA = true;

Serial.println("[OK] LoRa Radio channel active");

} else {

Serial.println("[CRITICAL] LORA ERROR! CHECK ANTENNA!");

// Infinite SOS loop if radio is dead

while (1) {

blinkSOS();

}

}

// Sending the starter packet

sendLoRa("SAT_BOOT_COMPLETE_SYSTEM_READY");

digitalWrite(LED_STATUS, LOW); // Turn off the LED, saving energy

delay(1000);

}

void loop() {

// 1. Continuous reading of GPS (to prevent buffer overflow)

while (gpsSerial.available() > 0) {

gps.encode(gpsSerial.read());

}

// 2. Telemetry timer (every 2 seconds)

static unsigned long lastTime = 0;

if (millis() - lastTime > 2000) {

lastTime = millis();

collectAndSendTelemetry(); // Collect and send data

}

}

// --- COLLECT AND SEND FUNCTION ---

void collectAndSendTelemetry() {

digitalWrite(LED_STATUS, HIGH); // Short flash when sending

// Variables for data (initialized to zero in case of error)

float temp = 0, press = 0, alt = 0;

float ax = 0, ay = 0, az = 0;

float busV = 0, current = 0;

int distance = -1; // -1 means "out of range"

int gas = analogRead(PIN_GAS);

// Collect data ONLY from live sensors (anti-freezing protection)

if (statusBMP) {

temp = bmp.readTemperature();

press = bmp.readPressure() / 100.0F; // hPa

}

if (statusMPU) {

sensors_event_t a, g, t;

mpu.getEvent(&a, &g, &t);

az = a.acceleration.z; // Vertical acceleration

}

if (statusINA) {

busV = ina219.getBusVoltage_V();

current = ina219.getCurrent_mA();

}

if (statusLOX) {

VL53L0X_RangingMeasurementData_t measure;

lox.rangingTest(&measure, false);

if (measure.RangeStatus != 4) distance = measure.RangeMilliMeter;

}

// --- EMERGENCY LOGIC ---

String sysStatus = "OK";

// If current > 500mA (short circuit or overload)

if (current > 500) sysStatus = "ALARM_PWR";

// If gas > 2500 (leak)

if (gas > 2500) sysStatus = "ALARM_GAS";

// If the battery is dead (< 3.3V)

if (busV < 3.3 && busV > 2.0) sysStatus = "LOW_BAT";

// --- ASSEMBLE PACKAGE (CSV) ---

// Format: ID,Time,Lat,Lon,Alt,Temp,Press,AccZ,Volt,Gas,Dist,Status

String packet = String(packetCount++) + ",";

packet += String(millis() / 1000) + ",";

// GPS (if there are no satellites, it will be 0.000000)

packet += String(gps.location.lat(), 6) + ",";

packet += String(gps.location.lng(), 6) + ",";

packet += String(gps.altitude.meters(), 1) + ",";

// Sensors

packet += String(temp, 1) + ",";

packet += String(press, 1) + ",";

packet += String(az, 2) + ",";

packet += String(busV, 2) + ",";

packet += String(gas) + ",";

packet += String(distance) + ",";

packet += sysStatus;

// --- SENDING ---

Serial.println(packet); // Duplicate to USB for debugging

sendLoRa(packet); // Send into space

digitalWrite(LED_STATUS, LOW);

}

void sendLoRa(String data) {

if (!statusLORA) return; // If the radio is broken, don't try to send

LoRa.beginPacket();

LoRa.print(data);

LoRa.endPacket();

}

void blinkSOS() {

// S (...)

for(int i=0; i<3; i++) { digitalWrite(LED_STATUS, HIGH); delay(100); digitalWrite(LED_STATUS, LOW); delay(100); }

delay(300);

// O (---)

for(int i=0; i<3; i++) { digitalWrite(LED_STATUS, HIGH); delay(400); digitalWrite(LED_STATUS, LOW); delay(200); }

delay(300);

// S (...)

for(int i=0; i<3; i++) { digitalWrite(LED_STATUS, HIGH); delay(100); digitalWrite(LED_STATUS, LOW); delay(100); }

delay(1000);

}

Code explanation:

Part 1. Libraries (Tools)

At the very beginning (#include) we take the tools out of the box.

C++

#include <Wire.h> // Wired communication (I2C)

#include <SPI.h> // Communication with Radio (SPI)

#include <LoRa.h> // Radio module control

#include <TinyGPS++.h> // Decoding GPS coordinates

... // Sensor-specific libraries

The point: Without these lines, the processor doesn't know how to talk to the hardware.

Part 2. Setting up Pins (Map)

This is where we tell the processor where we plugged the wires.

⚠️Important point for ESP32 S2 Mini:

C++

#define SDA_PIN 1

#define SCL_PIN 2

On a regular ESP32, the I2C pins are 21 and 22. But on our little S2 Mini, they're on pins 1 and 2. If you don't specify this, nothing will work.

Radio Setting:

C++

#define LORA_SCK 12 ...

We manually assign pins to the radio because they are not labeled as standard on this board.

Part 3. Health Flags (Survival System)

This is the main feature of the code.

C++

bool statusBMP = false;

bool statusMPU = false;

...

We create checkbox variables (true/false).

- At the beginning we consider all sensors to be dead (false).

- If the sensor responds "I'm here!" when turned on, we'll tick the true box.

- During the flight, we will only interview those who have a tick next to them.

Part 4. void setup() (Preflight check)

Runs once when power is applied.

1. POST (Power-On Self-Test) - Self-diagnosis:

C++

// Attempt to start the barometer

if (bmp.begin(0x76)) {

statusBMP = true; // If he responded, we check the "Alive" box.

Serial.println("[OK]");

} else {

Serial.println("[FAIL]"); // If it's silent, we write an error, BUT DON'T STOP!

}

We repeat this for each sensor. The satellite will launch even if only one of the five sensors is operational.

2. Critical Check (Radio):

C++

if (LoRa.begin(LORA_BAND)) { ... }

else {

while (1) { blinkSOS(); } // Infinite loop

}

The radio is the only component without which a satellite is useless (we won't find it or receive data). Therefore, if the radio is broken, the satellite goes into panic mode (flashing an SOS light) and doesn't fly anywhere.

Part 5. void loop() (Mission)

It's an endless cycle.

1. Listen to GPS:

C++

while (gpsSerial.available() > 0) {

gps.encode(gpsSerial.read());

}

The GPS module constantly babbles coordinates. We must listen to it constantly so as not to miss the moment when it locks on to satellites.

2. Timer (instead of delay):

C++

if (millis() - lastTime > 2000)

We don't use the delay(2000) (sleep) command because the satellite is deaf when it's asleep. Instead, we check the clock: "Has it been 2 seconds? If so, send the data."

Part 6. collectAndSendTelemetry (Building a Package)

The most complex function. It collects data into one pile.

1. Secure survey:

C++

if (statusBMP) { temp = bmp.readTemperature(); }

We only query the temperature if statusBMP == true. If the sensor is burned out, we don't waste time trying to reach it.

2. Alarm Logic: The satellite analyzes the data itself:

C++

if (current > 500) sysStatus = "ALARM_PWR"; // If the current is high, short circuit!

if (gas > 2500) sysStatus = "ALARM_GAS"; // If there is a lot of gas - Leak!

He will add this status to the end of the message.

3. Forming CSV (Tables): We glue all the numbers into one long line separated by commas: ID, Time, Coordinates, Temperature, Pressure... This is necessary so that on Earth we can open this data in Excel.

4. Sending:

C++

sendLoRa(packet);

The function sends a string into space through an antenna.